To update the RN2483 V 0.9.5 to V 1.0.0

This fix will currently only bring the RN2483 Module code to V 1.0.0. It will not correct the current boot loader issue which affects these devices.

Update the LoRa Mote Code to RC3

1] First the Mote Code must be updated to RC3. This can be done by reprogramming directly with a PICkit or ICD programming device.

If no tools are available please follow the below steps:

- Launch HIDBootloader.exe. (This is the same utility provided in the 7/14 release of the MLA. This GUI is also include with the LoRaMotePackage_RC3.zip)

- With the Mote Board powered down. HOLD either S1/S2 or both.

- Plug in the USB cable

- Red (D5) / Green (D6) Leds will alternate. These alternating lights are used to indicate the board is in the correct state.

- GUI should detect the attached Device.

- Press open icon. Browse and Select the LoRaMote_RC3_offset.hex file included in the LoRaMotePackage_RC3.zip

- Press the Write Icon

- Once completed. The Firmware has been updated.

- Once updated. The splash screen should now read “Microchip Mote LoRa Demo RC3 Code”

- The Mote will now communicate at 19,200 Baud Rate to the RN Module and allow the ability to perform a Boot Load to the device.

Update the LoRa RN Module Code

1] Connect the Mote to the PC using the USB-B cable.

2] Double Click and launch the LoRa.jar Executable. You should see the GUI attempting to find the device.

3] Select your RN Module device from the list on the left. Once selected you should see a (5) Tab menu selection; starting on the “LoRa Wan” tab.

4] Click on the DFU tab to begin a Device Firmware Update.

5] Click on the  icon to browse to the new Offset RN firmware Update. Setting should be left at the default values. Regardless of the Baud Rate selected, the Mote RC3 Code will only communicate with the RN Module @ 19,200. The Mote does not currently support selectable Baud Rates.

6] After you file is select. Click Update Firmware

7] Allow the Firmware to be updated. Once the device indicates success; -or- is dropped from the list. Power cycle the Mote board, by either disconnecting/reconnect USB cable; or using S3 switch if running from batteries. If the LCD screen reads “Valid Keys Required”.

If instead the screen reads “Mote Mode Starting ReRun PwrTst <s2>” the updated RN Module code likely did not succeed in loading. Follow the steps below in “Retrying the Boot Load Update”

Retrying the Boot Load Update

1] Because of a bug which was present in the RN2483 (v 0.9.5) boot loader. The attempt to update the RN Module code may not succeed on every attempt. RN2483 (V 1.0.0) or RN2903 (V 0.9.5) factory programmed, do not have this issue and will have no issues in the process.

2] If the 1st boot load did not work, the original RN2483 v 0.9.5 code would have been erased,; leaving only the boot loader on the device. Again in the LoRa Development Utility, above the device list, select the checkbox for “Bootloader Mode”

3] This will allow all available COM ports to be populated on the device list. Select the COM port which was enumerated for the LoRa Mote.

4] Only the DFU tab will be available. Follow steps 5-7 from Update the LoRa RN Module Code until the RN Module is successfully updated.

*Note: This COULD take multiple attempts.